- Iona Prep Physics Lab

- Light Bulbs Wired In Series

In this experiment you will set up a series circuit with three small light bulbs.  You will use a voltmeter and an ammeter to explore the relationships in this type of circuit.

Materials:

Power Supply

Ammeter

Digital Multimeter (used as voltmeter)

3 bulbs in sockets

Knife Switch

Hook-up wires

- Procedure

- Begin by drawing a neat schematic circuit using the power supply, knife switch, and all three light bulbs in a series circuit.  Indicate the voltmeter wired to measure the voltage across the entire circuit and the ammeter measuring the current through the circuit. Label the polarity on the ammeter.

- Have the instructor check your diagram.

- Obtain the necessary materials and construct the circuit according to your schematic.

- Before installing the bulbs, use the Digital Multimeter as an ohmmeter to check that each bulb has a fairly low resistance.  Very high resistance indicates that the bulb is burned out and will not function.

- Have the instructor check your circuit before applying power.

- Set the variable voltage to its minimum setting.

- Close the knife switch and adjust the voltage so that the bulbs are fairly bright.  The voltage across the circuit should be < 15 Volts DC.

- Record the voltage across the circuit and the current through the circuit.

- Draw another schematic diagram with the voltmeter reading the voltage across bulb 1.

- Before changing any of the wiring, open the knife switch.

- Now change the location of the voltmeter to read the voltage across bulb #1.  Record this value. (If possible, read this voltage using two different voltage scales. Do the readings agree?  If not, which one do you suppose is more accurate? Why?

- Repeat steps 9,10,11 to measure the voltage across bulb #2. Record this value.

- Repeat steps 9,10,11 to measure the voltage across bulb #3. Record this value.

Data:

Voltage across the entire circuit:

Voltage across bulb # 1

Voltage across bulb # 2

Voltage across bulb # 3

Conclusion:

What is the expected relationship among the voltage readings for this kind of circuit?

Do your measurements confirm or that relationship?