To Examine Voltage Drops in a Resistor/Capacitor (RC) Circuit

In this experiment you will connect two resistors in series as you did in the previous experiment.  However, rather than using a jumper wire to complete the circuit, you will use a capacitor.  Remember that the capacitor you are using is polarized and it must be connected observing polarity.

Procedure:

- Draw a schematic diagram for the power supply and two 1K ohm resistors wired in series with a 1000 microfarad capacitor between them.  Use the proper schematic symbols in your diagram and indicate the polarity of the capacitor.

- Construct a physical diagram on the Breadboard diagram.

- BEFORE TURNING ON THE POWER, attach a DC Voltmeter across the capacitor.

- Turn on the power and note how the voltage across the capacitor varies as a function of time.

- When the voltage across the capacitor appears to be stable, remove the voltmeter from the capacitor and measure the voltage across each of the resistors.

- TURN OFF THE POWER.

- Use a jumper wire to short circuit the capacitor, thus discharging it.

- Remove the jumper wire from the capacitor.

- Attach the voltmeter across one of the resistors.

- TURN ON THE POWER and note now the voltage across the resistor varies as a function of time.

- Turn off the power.

Questions:

- Describe how the voltage across the capacitor varied as time increased.

- Describe how the voltage across the resistor varied as time increased.

- How do you explain your observations?

- How do you think your observations would change if the 1K ohm resistors were both replaced with 10K ohm resistors?

- How do you think your observations would change if the capacitor were placed after both 1 K ohm resistors, rather than between them?